FLASHcode Decoder Ring

- A FLASHcode looks like this:

000000-000000-0

# Digit 12			Digit 1	Checksum

- Each digit is a Hex representation of 4 bits               B4  B3  B2  B1

- B4 has a weight of 8, B3 is 4, B2 is 2, B1 is 1, so if B4 and B2 in a digit are both enabled, that digit value is 10 or “A” (in Hex).

- The last digit is a checksum that is dependent on the value of the previous 12 digits.

- The definitions of the features that each bit represents are common for all subscribers* that support FLASHport (MTS, MCS, ASTRO Saber, ASTRO Spectra, ASTRO XTS3000, ASTRO XTS3500). ALL subscriber products do not support ALL features, however.  Consult price pages for options available for a particular subscriber

- The feature sets for Jedi series portables (MTS-2000, etc) are slightly different, and annotated as such.

- If two option numbers are separated by a slash, they represent first portable, then mobile options unless

- otherwise specified.

- Digit 12

B4	H15/H44   			H15 is Astro Data/ H44 is VRM500 for MCS2000 only

B3 – H14/G114			Digital ID Display

B2 – H260			 	250 Modes on W3 and W4 (150 or 160 channels for MTS2000)

B1 – Q806/G806			ASTRO Digital CAI (IMBE) Operation

Digit 11

B4 – H868/W298			Conventional Multikey and Analog Conventional MDC OTAR

B3 – H869/W969			Multikey Operation

B2 – H451				GE STAR

B1 – H101, G101, H169	Conventional Repeater Access (Analog Operation Only)(MDC1200 or Singletone)

Digit 10

B4 – H46				One Touch

B3 – H45				SmartNet Type I

B2 – H44/H29(MTS)		trunk Data Operation /H29 150 modes (mts2000)

B1 – G170/H43			Remote Monitor and Radio Trace

Digit 9

B4 – H42				Trunked variable talkgroup  ID

B3 – H41				Trunked badge ID

B2 – H40				RAC Software Option (Repeater Access, Single Tone)

B1 – H39				Selective Radio Inhibit

Digit 8

B4 – 					Future System Package

B3 – 					Future System Package

B2 – 					Future System Package

B1 – 					Future System Package

Digit 7

B4 – H38/G51			SmartZone Operation

B3 – H37/G50			SmartNet Operation

B2 – H36				StarSite Operation

B1 – H35/G48			Conventional Operation

Digit 6

B4 – 	H29				Single key Software Encryption Option

B3 – 	H07				Multikey software encryption

B2 – H915				International Radio (Internal Option)

B1 – Q181				StatAlert (MDC1200) Features

Digit 5

B4 –	Q351				MODAT

B3 –	Q352				Soft ID

B2 –	Q353				OTACR   (Over The Air Channel Reassignment)

B1 –	Q354				OTACS   (Over The Air Channel Steering)

Digit 4

B4 –	Q190				Channel change ID

B3 – H270/H738			MDC Status Message

B2 – Q241				ASTRO Ready (Analog Operation Only)

B1 – W947/Q947			APCO Packet Data

Digit 3

B4 – G182				Enhanced Radio Call Protocol

B3 – Q173/G173			SmartZone OmniLink Operation

B2 –	Q387				Conventional voting scan

B1 –	Q471				(MCS only) messaging (MDC on trunking)

Digit 2

B4 – Q361/G361			ASTRO 25 Digital Trunking Option

B3 –	W357 				Scan with VRS

B2 –	W995				zone/mode NJSP

B1 –					Future System Enhancement

Digit 1

B4 –					Future System Enhancement

B3 –					Future System Enhancement

B2 –					Future System Enhancement

B1 –					Future System Enhancement

Other notes:  Option Q241, Astro Ready, should actually be named 'Delete all Astro Modes' which

is an accurate description of what it does. A radio with this option does not have Astro digital mode options!

IMBE radios can NOT have software encryption.

With IMBE radios, MODAT can work on portables, not on mobiles.